TECHNICAL SHEET

| Designation | Desinfectant Wipes Medibase |  |
| --- | --- | --- |
| Reference(s) | The box of 100 wipes | 3131820 |
|  | The refill bag of 100 wipes | 3131821 |
| Product picture |  |  |
| Product description | Wipes impregnated with a disinfectant solution for cleaning and disinfecting non-invasive and non-submersible medical devices (small medical and dental equipment). |  |
| Characteristics and compositon | Less than 5% non-ionic surfactants, perfumes including limonene, disinfectants: Ethanol, Alkyl, Dimethylbenzylammonium chloride, Propan-2-ol. Wipes impregnated with a disinfectant solution for cleaning and disinfecting non-invasive and non-submersible medical devices (small medical and dental equipment). - Wipes particularly large, thick, strong and sweet. - Less than 5% non-ionic surfactants, perfumes including limonene, disinfectants: Ethanol, Alkyl, Dimethylbenzylammonium chloride, Propan-2-ol. ACTIVITE MICROBIENNE; Contact time. General treatments. Bactericidal; EN13727* - EN14561; 5min. EN14561: MRSA; 5min. Fungicidal; EN14562: Aspergillus fumigatus; 5min. Yeasticidal; EN13624 - EN 14562: Candida albicans; 5min. Tuberculocidal; EN14348 - EN 14563 : Mycobacterium terrae; 15min. Virucidal; EN14476* : HBV - HCV - Adenovirus Coronavirus - Novovirus - VRS - H1N1 - HSV1; 1min. EN14476: Polyomavirus; 1min. EN14476: Rotavirus; 15min. Additional uses. Bactericidal; EN1276; 30s. EN13697; 5min. Fungicidal; EN13697: Aspergillus niger; 5min. Yeasticidal; EN1650* - EN 13697: Candida albicans; 5min. *In dirty conditions. |
| Packaging | - The box of 100 wipes The refill bag of 100 wipes |  |
| Shelf life | 3 month from the day of opening |  |
| Indications of use | Ready-to-use wipes. Single-use wipes. When using the box for the first time: - Remove the cover, - Permanently remove the seal, - Thread the central wipe through the hole in the cover with clean hands and - Replace the cover. - Use the wipe on the equipment. - Leave on for 30 seconds to 15 minutes according to the antibacterial activity required. - Leave to dry in the open air. - Do not rinse. - Close the flap after each use to prevent the wipes from drying out. |  |
| Legal information | Medical Device of class IIA CE Marking 0459. G-MED Notified body. Manufacturer : FELT S.A |  |

Version 2.2						Updated 15/11/2022 by RG